## User Guide to OpenHENS

OpenHENS v0.5 is now publicly available as an open-source GitHub repository which can be accessed via Appendix A. OpenHENS is entirely free to use, as it relies solely on open-source solvers, modelling languages, and libraries, eliminating any software licensing costs. The package is released under the MIT License, allowing users to use, copy, modify, publish, sublicense, and/or distribute the software, provided that appropriate citation is given and the license text is included with any distribution. Future contributions from the community, such as the addition of new HEN problem types or features, are strongly encouraged.

OpenHENS includes a simple visualisation module, enabling the results to be viewed and shared easily. The visualisation module also offers options to show or hide layers of fidelity such as heat exchanger duties, stream temperatures, mixer nodes, and branch properties.

OpenHENS v0.5 is intended for users who are comfortable working with computers and terminal interfaces. The full release of v1.0 is expected to be more accessible to those with minimal experience in terminal programming. While modification of the source code is not required, the package is executed via a script interface that expects the user to specify values for nine options, listed in Table 1. All options have default values but to solve a new problem, the user must specify at minimum the input and output file paths.

- The input path should point to the stream data file for a new case.

- The output path designates where the solution data, parameter plots, and summary metrics will be stored.

It is recommended that users specify appropriate value ranges for ∆Tmin and dQdAmin for the problem, guidance will be provided here upon publishing the OpenHENS journal article. Additionally, the user may choose to override the automated stage selection in the PDM sub-models by specifying the stages explicitly. While the remaining four options can be modified, they are best left at their default values for initial users.

Table 1: Description of OpenHENS user options.

| Option | Description | Default |
| --- | --- | --- |
| input_folder | File path to the stream data CSV | examples/cases/Six-steam-Yee-and-Grossmann-1990-1.csv |
| output_folder | File path to results folder where outputs will be saved | Examples/results/Case 1 |
| min_dT_list | List of ∆Tmin values that define distinct PDM instances | 2, 4, 6, 8, 10, 12, 14, 16, 18 and 20˚C |
| min_dqda_list | List of dQdAmin values that define distinct TDM instances for each PDM | 0.5, 0.9, 1.3, 1.7, 2.1, 2.4, 2.8, 3.2, 3.6, 4.0 |
| stage_selection | Manual override for number of stages in each PDM sub-model | "automated" |
| tolerance | Numerical convergence tolerance | 1×10-3 |
| max_parallel | Number of models to solve in parallel (should be ≤ CPU cores) | 10 |
| best_solns_to_save | Number of N-best ESM solutions to retain | 10 |
| log_level | Logging level for controlling screen output | logging.WARNING |

To use OpenHENS within a code editor of the user’s choice:

- Install Python (version≥3.10).

- Download or clone the repository from GitHub. This can be done using a manual download, git or a graphical interface like GitHub Desktop.

- Create a conda environment to isolate package dependencies (e.g., Gekko, Pyomo). Detailed instructions are provided in the repository’s README.md.

- Activate the conda environment. This step must be repeated each time the code editor or terminal session is restarted.

- Install the package using pip install . .

- Prepare input and out directories:

- Save the custom stream data as a .csv file in examples/cases/ e.g. examples/cases/new_case.csv.

- Create a corresponding output folder in examples/results/ e.g. examples/results/new_case.

- This step may be skipped when solving one of the built-in benchmark problems.

- Set options. Open run.py and update the input_folder and output_folder paths. Modify the other parameters from Table 3.14 as required.

- Execute the script. Run run.py using the preferred terminal or code editor. Console messages will indicate progress of each stage of the synthesis method.

- Analyse the results:

- The output folder will contain:

  - Parameter variation plots

  - A spreadsheet summary of key metrics (solve time, lowest TAC solution, solution spread)

  - The N-best solutions saved as .pkl files.

- These saved .pkl files can be viewed using open_best.py which presents the grid diagram and variable values of each saved network.